SECTION 221200 - FACILITY POTABLE-WATER STORAGE TANKS

PART 1 GENERAL

- SUMMARY

- General

- Read this Section in conjunction with other related Sections, the General Requirements, the Design Drawings and the Contract Conditions.

- Performance Specified Work

- Refer to the Contractor's Responsibilities in the General Requirements for specific requirements.

- Supply, deliver, install and warrant the work in strict compliance with the materials and workmanship requirements of the Specification.

- Where required to prepare Shop Drawings, these shall be limited to final detailing of components, systems indicated on the Design Drawings, necessary to demonstrate their safe installation.

- Where products are offered by the Contractor for acceptance by the Engineer, provide full supporting documentation in respect of the complete system or installation.

- Included Systems/ Products

- Storage tank.

- SUBMITTALS

- General: Comply with the requirements of the General Requirements and submit the following.

- Post Contract Submittals

- Shop Drawings.

- Submit manufacturer's assembly-type Shop Drawings indicating dimensions, weight loadings, required clearances and methods of assembly of components.

- Product Data on materials and components for use.

- Supplementary Product Literature: Include a statement from the manufacturer for the design life of the system.

- List of tests included.

- Certified test data.

- Outline technical specifications reflecting proposed materials and systems.

- Method Statement.

- Quality Plan.

- Summary of deviations from the Specification.

- CLOSEOUT SUBMITTALS

- General: Comply with the requirements of the General Requirements and submit the following.

- Warranties.

- Operation and Maintenance (O&M) Manuals: Include component list with manufacturer's reference numbers, descriptions of materials and procedures for repairing and cleaning of finishes and cleaning frequency.

- QUALITY ASSURANCE

- Acceptable Manufacturers

- Subject to compliance with the requirements of the Contract Documents, acceptable manufacturers shall be firms regularly engaged in the manufacture of specified items with characteristics, sizes, and capacities required, whose products have been in satisfactory use under similar service conditions for not less than 10 years.

- Certification, test certificates under specified operation conditions shall be provided by manufacturer.

- Standards: In addition to the requirements indicated on the Design Drawings, or specified in the General Requirements and Specification, the Work shall be in accordance with the following standards, codes and relevant statutory requirements.

- Uniform Plumbing Code (UPC).

- International Plumbing Code (IPC).

- BSI Group, (BS) British Standards.

- International Organization for Standardization, ISO Standards.

- Provide testing and inspections in accordance with the General Requirements.

- Preconstruction Testing/ Reports

- Submit reports of independent tests demonstrating that the products and systems are in accordance with the specified performance requirements.

- Where test results for a material or product are not available, undertake testing to show compliance with the Specification at an independent testing laboratory acceptable to the Engineer.

- The provision of testing data or the carrying-out of tests does not relieve the Contractor of his responsibilities regarding the performance requirements, durability or service life requirements.

PART 2 PRODUCTS

- SYSTEMS/ PRODUCTS

- Roof Storage Tank

Polyethelyne tanks for spec. refer to BOQ

- RCC Water Storage Tank

- The main water storage tank shall be reinforced concrete construction and shall be of the capacity and dimensions as indicated on the Design Drawings.

- They shall comply with the relevant Local Authority standards for internal linings.

- Tanks shall be complete with the following:

- Openings with sealed manhole cover for each valve/ accessory.

- Manholes shall be double sealed manhole covers. With watertight hinged cover and locking device.

- Internal stainless-steel ladders.

- Inlets, outlets over flows, drains and insect proof vent cowls as required on the Design Drawings.

- MATERIALS

- General

- Concrete structures shall be designed in accordance with BS EN 1992-1-1 or Jordanian codes

PART 3 EXECUTION

- EXAMINATION

- Verification of Conditions

- Examine areas for compliance with requirements for installation and conditions affecting performance of the Work. Identify conditions detrimental to a proper and timely completion. Proceed with installation only after unsatisfactory conditions have been corrected.

- INSTALLATION

- General

- All tanks shall be installed in accordance with the manufacturer's recommendations.

- All cisterns shall be installed on either flat continuous foundation or close center beams in accordance with the manufacturer's recommendations for flatness and deflection.

- SITE QUALITY CONTROL

- General

- Testing shall be carried out on completion of installation and to be completed within 10 working days of assembly.

- The testing of panels and sealant shall be as listed by Water Research Center, UK conforming to water fittings by-laws scheme.

END OF SECTION