- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes

          - Equipment necessary for the best operation of the Queue Management System (QMS) within the building such as system’s server, ticket dispensers, calling units, zone displays with built in speakers, counter displays, system rack, network cabling, management software…etc.

      - REFERENCES

        - The system shall comply to BS, IEEE, ISO/IEC standards.

        - Requirements and requirements of Local Authorities having jurisdiction.

        - Local Codes and Regulations

        - All equipment and systems installed under this Contract shall meet following specifications on electromagnetic compatibility:

          - Interference immunity according to BS EN 61000: Part 6-3

          - Emitted interference according to BS EN 61000: Part 6-2

      - SYETM DESCRIPTION

        - The QMS system should be provided to emergency department as shown on drawings.

        - The system should be digital type, consisting of system server, ticket dispensers, zone displays with built in speakers, counter displays management software, system cabling, staff calling units, ..etc.

        - The QMS system should be interfaced to fire/ voice alarm system, IT network, BMS and Public Address System.

      - SUBMITTALS

        - System schematic diagram as proposed describing locations and types system components.

        - Method Statement for system installation, testing, operation and commissioning.

        - Schedule of all equipment with model number, names of manufacturers and country of origin.

        - Product data sheets highlighting the selected model of proposed equipment. Product data sheets for each and all system components.

        - Full system design calculations including voltage drop calculations.

        - Copy of contract specifications complete with clause wise compliance statement.

        - Pre-qualifications of the specialist systems integrator and the proposed manufacturer List of projects where systems Integrator has implemented products from the same manufacturer.

        - Demonstration of system features and capabilities in an existing installation of similar complexity, either in Karbala or abroad.

        - Provide samples of all proposed devices, fittings, outlets, accessories, …etc. and as required by the Engineer together with the above submittal for approval of the Engineer. It's the contractor's responsibility to get the samples approvals without any extra time extension.

        - Workshop drawings:

          - Layout drawings complete with cable sizing, cable routing, containment sizing, location of equipment, power requirements and data ports for system components, etc and installation details of active and passive distribution components.

          - Drawings shall also indicate requirements of back boxes, pull boxes, access panels, data, power etc.

          - The above information shall be provided to the Contractor on time as per the project programmer for providing containment and builder's works.

        - As-built drawings: Submit the number of sets of final as-built schematic drawings as indicated in the contract documents highlighting locations all components included in the system complete with labels.

        - Operation & maintenance manual: Submit 6 set of Operation & Maintenance manuals complete with preventive maintenance schedules, procedures and troubleshooting procedures.

        - Detailed list of Equipment & Software following the BOQ format below:

| Item No. | Item Category | Area | Type | Vendor | Description | Model # | Country of Origin | Unit of Measurement | Qty | Ref # | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

        - Detailed list of Materials following the BOQ format below:

| Item No. | Item Category | Area | Type | Vendor | Description | Model # | Country of Origin | Unit of Measurement | Qty | Ref # | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

          - All civil, MEP and support works, including and not limited to:

            - Containment.

            - Cabling.

            - Scaffolding and access ladders.

            - Labor.

            - Power supply.

            - Other tools & services.

          - Unless advised otherwise, Contractor is self-supportive and should include for his offer his own materials, tool, services, office, and any requirement necessary to complete this project.

          - List of samples to be provided.

          - Coordination with other parties.

            - Method statement.

            - Scope of work matrix for all works listed in this Contract Deliverables Section.

            - Communication protocol.

          - Outline risk assessment for all major aspects of the works.

          - General method statement/risk assessment on approach to day to day responsibilities.

          - List of Spare parts to be provided for minimum 10 years.

          - Item by Item commentary on technical requirements.

          - The Contractor shall comply with all specifications described in the Technical Specification sections herein. System specifications proposed should reflect each of the system components in question, functionality desired by the system component and the how the component integrates with the centralized system. The Contractor shall add 3 columns beside each item to state the following:

            - First column is ‘Bidder’s Compliance’:

              - Contractor shall specify here one of the below:

                - Y: proposal does comply with specification/ requirement.

                - N: proposal does not comply with specification/ requirement.

                - N/F: proposal does not fully comply with specification/ requirement.

                - N/A: specification/requirement is not applicable to the project.

            - Second column is ‘Reference’:

              - Contractor shall refer each compliant item to the related section of proposal.

              - Component data sheet or reflected electrical/ceiling/wall plans layout drawing.

            - Third column is ‘Bidder’s Comments’:

              - Contractor shall note down any further description, comment, feedback, concern, etc. on each item.

              - Contractor shall note down any further description, comment, feedback, concern, etc on each item.

          - Vendor certificates.

            - Company certification by vendor as authorized dealer.

            - Personnel certificate by vendor as factory-trained.

            - Product quality assurance certification by vendor.

          - Vendor list and selection criteria for all items, vendors should adhere to the following criteria:

            - Company established at least 10 years ago.

            - Have references of at least 20 projects globally of similar scope & complexity.

            - Based in Karbala/ Iraq or neighboring countries since at least 5 years.

            - Have references of at least 5 projects in Karbala/ Iraq or neighboring countries of similar scope & complexity.

            - Have a local support office.

            - Have a local warehouse for spare parts stocking.

          - Proposed subcontractors identified and scope of work for each.

          - Confirmation of guarantee periods for equipment and guarantee terms & conditions.

          - Confirmation of warranty periods for equipment and guarantee terms & conditions.

          - Maintenance & support offering – sufficient details.

          - Other supporting information that adds value to the proposal.

        - Bidder’s Compliance Letters.

          - Conformity of bid to requirements – sufficient technical details.

          - Conformity to agree on Confidentiality Agreement.

          - Letter of Bidder’s Compliance with All Specifications.

          - Conformity of responsibility to secure Compliance with related government/utility authority.

        - List of Proposed Personnel with CVs & Responsibility Matrix.

          - Relevant experience for key team members.

          - Other supporting information that adds value to the proposal.

        - Schedule of Works: Bidder shall compile a schedule of works and include a project milestone as per Master Schedule of Works provided. It is up to the bidder to create a schedule that best incorporates all works.

        - Handover & Close-Out Documentation:

          - As Built Drawings.

          - End User Manuals.

          - Maintenance Manual.

          - Operations Manual.

          - Plans as mentioned above.

          - S/W license, update and upgrade. procedures.

          - Other supporting information that adds value to the proposal.

      - CLOSEOUT SUBMITTALS

        - Project Record Documents: Record actual locations of outlets, devices, and cable routing.

        - Operation and Maintenance Data: Submit instructions for setting and tuning channels, and for basic trouble-shooting procedures.

      - QUALIFICATIONS

        - System components shall be manufactured by firms that are regularly engaged in the production of QMS components and auxiliary equipment similar to that required for this project and that have been satisfactory service for at least 10 years.

        - The firm installing the system shall have had prior successful installation experience with QMS systems similar in scope to the system of this project.

        - The system shall be engineered, commissioned and serviced by manufacturer's authorized systems integrator employing factory trained personnel for commissioning and after sales support. Materials and equipment shall be the catalogued products of manufacturers regularly engaged in production of access control equipment and the products proposed shall be the latest versions commercially available at the time of the bid.

        - The manufacturer shall be ISO-9002 and ISO 9001 certified for Quality Assurance in Production, Installation and Servicing.

        - The systems integrator shall be an authorized partner to the manufacturer and the partnership shall be at least two year old. The manufacturer systems integrator combination shall have implemented at least three projects of similar complexity in Karbala/ Iraq or neighboring countries.

        - The specialist shall obtain and submit a written undertaking from the manufacturer that the spare parts of the system proposed will be made available upon request for at least 10 years from the date of substantial completion. In addition, the manufacturer shall undertake to ensure forward compatibility of the proposed equipment for at least 15 years.

      - QUALITY ASSURANCE

        - The Contractor shall post a note, at the main system components, to indicate the name, address, and telephone number of the firm to call when service is necessary.  Note shall be mounted in a metal frame attached to the equipment case.

        - Operation of the QMS system shall be demonstrated to the Engineer to prove, under normal conditions, that received signal complies with this specification.

      - SPARE PARTS

        - The Contractor shall respond to trouble calls with a competent repair person at the project site within 24 hours; Contractor shall also maintain a full inventory of replacement parts so that all routine repairs can be completed within 24 hours.

        - Supply as maintenance stock, consumable devices, components as recommended by Supplier, but shouldn’t be less than two units of each type/ rating of installed consumable material/ component/ device, incase less than three numbers of devices/ material are installed, minimum of one unit of each type/ size/ rating of each device/ material shall be provided as spare stock.

      - MAINTENANCE SERVICE

        - Maintenance Contract: Obtain an undertaking by the equipment manufacturer and his authorized local representative to provide an annual maintenance contract, available after substantial completion of the work, covering routine service and emergency service by trained employees, and to ensure supply of necessary standard spare parts.

        - Initial Maintenance Service: Beginning at Substantial Completion, provide (12) months full maintenance service by skilled, competent employees of the Installer. Include monthly preventive maintenance, repair or replacement of worn or defective components, cleaning, and adjusting as required for proper operation. Use parts and supplies as used in the manufacture and installation of original equipment.

          - Perform maintenance, including emergency callback service, during normal working hours.

          - Include 24-hour-per-day, 7-day-per-week emergency callback service. a. Response Time: 2 hours or less.

      - WARRANTY

        - Submit written guarantee signed by the manufacturer and the contractor for the period of (1) year from the date of substantial completion. The guarantee shall cover the repair and replacement of defective materials and workmanship as directed by the engineer except where any specific warranty from a supplier or manufacturer extends for a longer time.

      - TRAINING

        - Engage a factory-authorized service representative, whose credentials are approved by the Engineer to provide operation, adjusting, trouble shooting, & servicing system administration training for the proposed system to three Engineers. Training shall be sufficient enough to enable the engineers to carry out day-to-day operation, maintenance and software and hardware administration of the system without or with minimal assistance from the vendor’s technical staff.

        - Training shall be carried out to an approved training manual.

        - Training shall have both class room session and hands on session.

- PRODUCTS

      - GENERAL

        - The complete system and cables shall be manufactured preferably by a single manufacturer.

      - QMS SERVER

        - The QMS Server shall manage the operation of the entire queue system. It shall be a multitasking server; one PC controlling each floor QMS system. It uses RS485 and Ethernet links to control the queue devices.

        - When a customer requests for a ticket, the QMS Server logs down the information and assigns a suitable ticket number. This number together with other information such as date, time and currently served number is sent to the Ticket Printer.

        - When a staff requests for a customer, the QMS Server looks up the appropriate service queue and responds with the next-in-line customer ticket number and updates the Main Display and Voice Module.

        - The screen of the QMS Server shall show on-line information about the queue, such as the number of people waiting for various services, the waiting time and the numbers of counters currently open. It even computes the number of counters required to cope with the current situation if the preferred service quality is to be maintained.

        - The QMS Server shall store all queue data in its hard disk which facilitates subsequent analysis of customer arrival patterns, service quality, staff productivity and etc. In the event of a power failure, it can recover its data from the hard disk and continue from where it was interrupted.

        - Required PC Minimum Specifications:

          - CPU:			 Pentium 4 or Higher.

          - RAM Memory:		 512 MB RAM

          - Drive:			 1.44 MB FDD / CD ROM Drive

          - Hard Disk Drive:	 80 GB or Higher

          - Video Card:		 SVGA

          - Monitor:		 19-inch LCD Monitor

          - Operating System:	 Windows XP Prof., mouse and keyboard

        - Information Display: QMS Server software, shall be provided for maximum information handling and for the better counter staff management. QMS Server shall be capable to send many information on the Branch Manager's computer online in the web browser page format.

        - Log In/Out Password

          - QMS system shall provide LOG IN PASSWORD facility to reduce the job of Manager or Administrator to program the staff work schedule. All the counter staff shall have unique 4 digit password, which helps them to log in to their account, at the time they start working on the counter.

          - This option shall provide flexibility to the Manager, to appoint any staff member on any counter and with the help of their password their activity and performance will be recorded in their account only.

        - Software User License: QMS Server Software shall be able to handle more than one Ticket Printers, to facilitate the use of one QMS Server at each floor to control all the ticket printers, counters, and other items attached. Software licensing shall be provided for each system for the benefit of the Engineer.

        - Corporate Report Generator: The Corporate/ the Engineer Report Generator software shall be provided for each system for the comparison reports for all tellers/ customers service providers in the region. It shall contain a list of reports appropriate to corporate analysis, the data required by this report generator will have to come from tellers / customers’ service providers counters/ desks.

      - QMS TICKET PRINTER / DISPENCER

        - The ticket dispenser should have 17-inch LCD enables to display up to 10 task (group) buttons in one page.

        - The ticket dispenser should capable to display unlimited number of tasks/services.

        - The ticket dispenser should arrange different tasks at a time, it is not necessary to take a ticket to each one, as up to three tasks can be chosen on one ticket. The software guarantees that a customer cannot be called to more than one counter at the same time.

        - The ticket dispenser should have a high speed (220 mm/s) 3-inch thermal printer.

        - This ticket dispenser should support the following features:

          - Three tasks on one ticket

          - Handling unlimited number of tasks

          - Choosing the clerk by digital photo

          - Five different languages

          - Ethernet connection to the server

          - Paper low detection with alert system

          - Stand floor mounted

      - QMS TICKET FORMAT

        - QMS must be user friendly, for users, staff and customers as well, with the maximum information on the waiting ticket itself, not only the waiting number, date, time, company logo, and name of the service/ category, but also information messages per management instructions can be put on the waiting ticket like:

          - Information for new services.

          - Information about the change in working hours timing, e.g. in Ramadan, summer/winter.

          - Information about the counters serves like: 
"Your number will be served on counters 1 to 5" 
(It will help the customers to set near the same counters)

          - Massage length shall be as per management requirement, in Arabic or English or in both.

        - Once any customer touches the required service on the ticket printer screen, ticket printer will print the waiting number ticket, and an automatic cuter will cut the paper after finishing the printing.

        - Back side printing of the ticket roll:

        - It shall be possible to print any message on the reveres side of the thermal ticket printer roll, it shall be in any color or in the multicolor as per the Engineer requirements.

      - QMS COUNTER/STAFF TERMINAL CALLING UNIT

        - Icons and colors shall be used to further assist the operator in using the Counter Terminal. A full alphanumeric LCD (Liquid Crystal Display) panel is used to show queue number and other information. A buzzer provides an audible signal to acknowledge the user's command or indicate abnormal conditions such as wrong data entry or new customer arrival.

        - The Counter Terminal shall be used by the staff to call the next customer. The ergonomic Counter Terminal shall provide sophisticated yet simple-to-use functions. A total of 31 keys are available, divided into three sections commonly used keys, infrequently used keys, and numeric keys. In the first group, the most important key is an extra-large key spanning the width of the Counter Terminal for calling the next customer. This particular key is the only key pressed under normal operation.

        - Specifications:

          - Display Element:	 LCD (Liquid Crystal Display)

          - Display Size:		 1 line by 16 digits

          - Digit Height:		 7 mm

          - Audio Output:		 Buzzer

          - Input Voltage:		 V DC 12.0

          - Power Consumption:	 0.08 ml. W

          - Casing:			 Low profile molded plastic

          - Dimension mm:		 90 x 133 x 28

        - In case of long handling time and to avoid embracing for the customers as well as staff, counter terminal will beep-up a buzzer to make alert the counter staff, so that counter staff can finish the customer's job quickly and control the increasing handling time.

        - To make the things more efficient and quick, this handling time increasing report will be sent to the Manger, in a pop up format and Manager can check with the counter staff if they need help.

        - Counter Terminal is also able to provide an audible signal (buzzer) for the acknowledgement of counter staff as a new customer gets waiting ticket.

        - In case if waiting time increase for the respective service, counter staff will get the buzzer signal to be alert to control the waiting time and increase the working speed, the same information will go the branch Manager in pop up format and Manager can tack any quick action to control the situation.

        - Whenever counter staff needs Manager's help, there is a key to push and there will be a pop up on the Manager's computer screen with the counter number, for help message.

      - QMS COUNTER DISPLAY

        - The Counter Display is a local display directly connected to the Counter Terminal to show the queue number currently being served at the counter. It shall connect to the Counter Terminal by a 4-pair UTP cable and shall be typically mounted at the counter top. The display shall enclose in a chromed aluminum casing. It consists of 4 digits of 4" solid 7-segment LED display in red, amber or green. An electronic chime shall incorporate and sounds each time a new number is being displayed.

        - At calling, the ticket number is blinking e.g.: for 30 seconds (blinking time can be adjusted using the management software) on the display

        - Specifications:

          - Display Element:	 7 – Segment LED

          - Digit Height:		 100 mm

          - Color:			 Red, Green or Amber

          - Audio Output:		 Chime or Buzzer

          - Input Voltage:		 12 VDC

          - Input Current:		 0.6 A

          - Casing:			 Chromed Aluminum

          - Dimension mm:		 395 x 155 x 35

      - QMS WAITING AREA/TERMINAL DISPLAY

        - The Waiting Area Terminal Display shall be identical to Counter Display in terms of functionality. But dot matrix LED modules are used instead of 7-segment modules to display alphanumeric characters. Arabic characters shall be supported. It is connected to the Counter Terminal by a 4- pair UTP cable and it shall enclose in a chromed aluminum casing. The LED height shall be 4" and available in red, amber or green color. An electronic chime shall be incorporated and sounds each time a new number is being displayed.

        - The waiting area/ terminal display should blink for around ten seconds after the call (The blinking time can be adjusted using the management software).

        - Specifications

          - Resolution:		 16 X 64 dots

          - Display Element:	 Dot matrix LED

          - Digit Height / mm:	 64

          - Color:			 Red, green or amber

          - Audio Output:		 Electronic chime

          - Input Voltage:		 12 V DC

          - Input Current:		 1.20 A

          - Casing:			 Chromed aluminum / Chromed Steel

          - Dimension:		 322 (L) x 125 (H) x 35 (D) mm

          - PCB physical size:	 150 (H) x 480 (W) x 45 (D) mm

          - LED display area:	 120 (H) x 480 (W) mm

      - QMS VOICE MODULE

        - The Voice Module shall be a value-added feature of the Queue Management System. It shall produce a synthesized digital voice used to announce a customer's queue number and counter number when a queue number is being displayed. The announcement can be in Arabic and/ or English. This feature shall provide a cordial and efficient way to serve customers.

        - Also a public address system can be attached with this voice module, with ceiling speakers, and desk microphones for any individual announcement, the public address system will have the over-ride facilities.

        - Specifications

          - Type:			 Digital Voice Synthesizer using CVSD technique

          - Language:		 Arabic and/or English

          - Inputs:			 Microphone and Line-in

          - Output:			 Line-out

          - Input Voltage:		 12V DC

          - Power:			 25 W

          - Dimension:		 280 x 70 x 190 mm

      - QMS POEWR SUPPLY HUB

        - This is a power supply unit which also acts as a data distribution hub. It is equipped with twelve RJ45 ports. It provides the 12V DC operating voltage as well as the RS485 signals to the queue devices. One of the ports can be used for connecting to another PS12 to form a highly distributed system. This power supply box has steel cover for the safety and the security.

        - Specifications

          - Input Voltage:		 220V AC

          - Output Voltage:		 12V DC

          - No. of RJ45 Ports:	 12 Ports

          - Type:			 Switching

          - Max Output Power:	 90W

          - Dimension:		 165(W) x 250(L) x 65(H) mm

- EXECUTION

      - SUPERVISION

        - All works associated with implementation of the Queue (QMS) Systems shall be carried out under the direct supervision of a representative of the approved specialist systems integrator. Installations shall be carried out by skilled technicians.

      - WORKSHOP DRAWINGS

        - Detailed engineering shall be the carried out by a factory trained engineer working with the approved specialist systems integrator. Workshop drawings shall detail all cable types, conduits, backbox / power requirements, mounting details etc.

        - Workshop drawings submitted for the engineer’s approval shall be duly signed and stamped by the specialist systems integrator.

        - Detailed system schematic diagram showing cable sizes and types, exact locations of devices, PC etc. shall be submitted along with the workshop drawings.

      - COORDINATION

        - The supervisor of installation shall be fully responsible for coordination with specialist integrators of other systems to which the system has an interface or integration.

      - LABELING

        - A labeling scheme shall be submitted for approval of the engineer. All cables and devices shall be labeled to the approved scheme.

      - CONTAINMENT

        - All wires and cable shall be installed in rigid PVC conduits or metallic cable trays. Sizing of containment shall be carried out during shop drawing to be carried out by the specialist. Containment shall be sized such that a 30% spare capacity is available for any modifications to the system after handing over.

      - GROUNDING

        - Each piece of electronic equipment shall be properly grounded before energizing. All wire shields shall be grounded at the hub end only, not at the remote or device end. Grounding system components, conductor and cable shields to eliminate shock hazard and to minimize ground loops, common-mode returns, noise pickup, cross talk, and other impairments.

      - TESTING AND COMMISSIONING

        - All tests shall be carried out according to a detailed procedure duly approved by the engineer and as stated in section 26 01 26. The tests shall be carried out in phases.

        - Cables are to be tested for any damage occurred during transportation to site and/or during the course of pulling-in and laying.

        - Pre-testing: Upon completing installation of the system, align, adjust and balance the system and perform complete pre-testing. Determine, through the pre-testing, the conformance of the system to the requirements of the drawings and specifications. Correct deficiencies observed in pre-testing. Replace malfunctioning or damaged items with new and retest until satisfactory performance and conditions are achieved. Prepare forms for systematic recording of acceptance test results.

        - Report on Pre-testing: After pre-testing is complete, provide a letter certifying the installation is complete and fully operable, including the names and titles of the witness to the preliminary tests.

        - Final test notice: Provide a 10-day minimum notice in writing when the system is ready for final acceptance testing.

        - Resetting: Correct deficiencies indicated by tests and completely retest work affected by such deficiencies. Verify by the system test that the total system meets the specifications and complies with the applicable standards.

- END OF SECTION