SECTION 23 8127

DX SPLIT AIR CONDITIONING UNITS

PART 1 GENERAL

- SCOPE OF SECTION

A. This technical specification establishes the quality of materials and workmanship to be used in the supply and installation of the packaged and Split air conditioning units, all the specification must comply with BOQ.

- WORK INCLUDED

- Provision of all labor, materials and the performance of all operations in connection with the supply and installation of the packaged and room air condition as specified herein and shown on the drawings.

- Coordination: The Contractor shall be responsible for proper coordination of the work of all trades.

- QUALITY ASSURANCE

- Manufacturers: Firms regularly engaged in the manufacture of packaged and room air conditioning equipment and fittings whose products have been in satisfactory use in similar service for not less than 10 years.

- Installer: Firms regularly engaged in the installation of packaged and room air conditioning equipment of a similar quality and scope as this project for at least 5 years.

- Local Agent: Local agent shall have at least 10 years continuous presence in the local market and shall demonstrate full maintenance and after sales capabilities to the satisfaction of the engineer and employer.

- All equipment shall be run tested in accordance with the following procedures prior to leaving place of manufacture.

- A choke test carried out on the refrigerant piping to detect obstacles

- The pipe work shall be tested to 38bar.

- Electronic leak testing shall be carried out to ensure maximum system refrigerant containment.

- System vacuum test to 2 Torre

- Electrical tests shall include flash testing at 1440V AC to ensure that current leaks above 5mA are detected, mugger test at 500V DC to ensure resistance levels are above 10 mega Ohm and earth continuity tests

- All units shall be AHRI certified

- APPLICABLE CODES AND STANDARDS

- The room air conditioners shall comply fully with the latest relevant International and British Standards in all respects.

- The following are the most commonly used standards associated with ducted split air conditioners; however the Contractor shall ensure that all applicable standards are complied with, whether listed here or not.

Household and similar electrical appliances. Safety Particular requirements for electrical heat pumps, air- conditioners and dehumidifiers

BS EN 60335-2-40

BS EN 14511-1

NEC Article 44 ISO R859 ARI 210 ARI 270 ARI 360

Air conditioners, liquid chilling packages and heat pumps with electrically driven compressors for space heating and cooling Terms and definitions

Air Conditioning and Refrigerating Equipment

Testing and Rating Room Air Conditioners

Unitary Air Conditioning Equipment

Sound Rating of Outdoor Unitary Equipment

Commercial Industrial Air Conditioning Equipment.

- SUBMITTALS

- Drawings refer to 15010

- Products: Submit full manufacturer's data for every item.

- OPERATION AND MAINTENANCE DATA

A.	Comply with 15010

- WARRANTY

A. Furnish 60 months manufacturer warranty for compressors and complete assembly.

PART 2 PRODUCTS

SYSTEM DESIGN

Split-System Heating and Cooling Units:  Self-contained, packaged, matched factory-engineered and assembled, pre-wired indoor and outdoor units; UL listed.

INDOOR UNITS FOR DUCTLESS SYSTEMS

Indoor Units:  Self-contained, packaged, factory assembled, pre-wired unit consisting of cabinet, supply fan, evaporator coil, and controls; wired for single power connection with control transformer.

OUTDOOR UNITS

Outdoor Units:  Self-contained, packaged, pre-wired unit consisting of cabinet, with compressor and condenser

DX SPLIT SYSTEM AIR CONDITIONERS

- Direct Expansion (DX) split system air conditioning units shall be installed as indicated on the drawings and shall be complete with all required refrigerant piping, temperature controls and all other necessary ancillary items.

- The units shall deliver the design cooling capacity at the external ambient specified. The units shall be suitable for continuous operation with external ambient temperature at 47.7°C.

- Controls shall be factory wired and completely enclosed within the unit. All operating controls shall be located in a single area. Adjustable thermostats shall automatically cycle the compressor to maintain space conditions and the sensing element shall extend across the complete face of the cooling coil.

- Condensate removal shall be by means of gravity drainage.

- Evaporator and condenser coils shall be of copper tube construction with aluminum fins and additional anti corrosion coating suitable for salty spray atmosphere.

- Compressor motor shall be 2 poles, permanent split capacitor type protected against both thermal and electrical overload and fitted with an inverter for output modulation.

- Filters shall be washable type easily accessible and shall cover the full unit area of re-circulated air. Air filters may be nylon fiber, glass fiber cellular plastics material and shall have a minimum efficiency of 80% when tested in accordance with BS 2831 Test Dust No. 3.

- Refrigerant stop valves which incorporate a spindle gland shall be serviceable with the valves "in situ".

- Gas line insulation shall be carried out using min.13 mm thickness of a cell, foamed plastic, tubular pipe insulation. Tape all joints to form a good vapor seal, then wrap with glass cloth and paint with two coats of approved vapor seal.

- The evaporator/fan coil section shall be ceiling, wall or floor standing and complete with concealed control panel and finishes as indicated on the drawings.

- The control panel shall have at least the following functions:

- On/Off/Cool Control

- Low/High cooling control

- Adjustable Thermostat

- Air discharge direction control On/Off

PART 3 EXECUTION

- SELECTION

- The equipment shall be selected for the design conditions and as recommended by manufacturer.

- PRODUCT STORAGE AND HANDLING

- All products shall be delivered in manufacturer's original protective packaging.

- All products shall be inspected at time of delivery for damage and for compliance with Specifications.

- All products that are found to be damaged, or not in accordance with the Specifications shall immediately be repaired or removed from the site and replaced. Repairs shall not be undertaken before Engineer's review of the Contractor’s proposed action.

- All products shall be handled and stored as recommended by manufacturer to prevent damage and deterioration.

- The Contractor shall supply handling equipment such a lifting beams, reinforced canvas slings, protective puddings, struts, cradles, etc., required to handle products without damaging hardware or linings and coatings.

- COORDINATION

- Requirements of Section 15050 shall be referred to and complied with in all respects.

- General : Install multi split cooling units in accordance with Manufacturer's Installation Instructions. Install units plumb and level, firmly anchored in locations indicated, and maintain Manufacturer's recommended clearances.

- INSTALLATION

- Install all components of the system and related to it according to the manufacturers instructions.

- All units mounted externally shall have a raised concrete base with a minimum height of 100mm above surrounding surfaces. The base shall be sloped to provide natural drainage and ensure that pounding does not occur under the unit.

- Each unit of whichever type shall be complete with a local electrical disconnect switch.

- The units shall be of the sizes, capacities, duties and types indicated on the drawings and shall be installed in strict accordance with the manufacturers requirements.

- Each outdoor unit shall be installed on a vibration isolation base as per relevant specification section.

- ELECTRICAL WORKS

- Electrical Wiring : Install electrical devices furnished by manufacturer but not specified to be factory-mounted. Furnish copy of Manufacturer's wiring diagram submittal to Electrical Installer. Verify that electrical wiring installation is in accordance with Manufacturer's submittal and installation requirements of Division 26 and all other sections. Do not proceed with equipment start-up until wiring installations is acceptable to Equipment Installer.

- COMMISSIONING AND SYSTEM DEMONSTRATIONS

- The services of a competent, factory trained engineer shall be provided by the Contractor to supervise the following: All expenses in connection with these shall be allowed for by the Contractor.

- Inspection and testing of the units, prior to start up.

- Start up and commissioning checks, to be conducted only during the summer season.

- A minimum of one week's written notice shall be given and the Owner’s written consent obtained, prior to such demonstration / training.

- Start up split air-conditioning units, in accordance with manufacturer's start up instructions. Test controls and demonstrate compliance with requirements. Damaged or malfunctioning controls and equipment shall be replaced.

- CORROSION PROTECTION

- The structural steel frame work and the sheet steel panels of the evaporator and condenser units shall be protected against corrosion by appropriate means; Manufacturers’ recommendations shall be sought.

- Heat transfer surfaces exposed to atmosphere, such as condenser coils shall be protected against corrosion from atmospheric conditions in and around the project site; by manufacturer’s proprietary coatings.

END OF SECTION